MERCHANT MARINE ACADEMY OF MACEDONIA

SCHOOL OF ENGINEERS

Course: Maritime English

Academic year: 2021 – 2022           		           Name:

Exam period: September			       	Student's number:

Semester: ST'						Date: 21/09/2022

Instructors: A. Birbili, M. Tsompanoglou		Exam paper grade:

FINAL EXAM

1. Put an appropriate word from the list in the gaps that follow. There is one (1) extra word.   (15 p.)

centrifuging    cylinder    removable    loads    temperature     irregularities     crude       permanent

micro-seizure     emissions     ovality     exhaust      welding     washing     monitoring     expansion

-- COW stands for _______________ oil _______________.

-- Scuffing is a form of localised _______________ between the particles of piston rings and the liner surface.

-- _______________ in the liner and piston rings coming into contact during operation are the cause of _______________.

-- One way of _______________ the condition of the cylinder liner is checking its ____________.

-- _______________ is the most effective way of removing water and dirty particles from fuels.

-- _______________ insulation is not replaceable. Its initial cost is less than _______________ insulation.

-- An ______________  valve regulates the refrigerant in order to maintain the correct _______________.

-- With severe cold corrosion that is not addressed by correct _______________ oil, a liner can be damaged beyond useful life in less than 1,000 hours.

-- The latest generation slow-speed engines combine lower ______________ and better fuel economy by running at part or very low _______________.

2. Complete the sentences with the correct form of the words in parentheses.  (20 p.)

-- Fibrous insulation materials are suitable for _______________ (vibrate) environments.

-- _______________ (abrade) wear is sometimes caused by _______________ (catalyst) fines.

-- Combustion ____________ (efficient), among other factors, influences the wear rate of the cylinder liner.

-- The drier removes any _______________ (moist) from the refrigerant.

-- Heat _______________ (lose) may be reduced a lot with proper insulation.

-- The recommendation for a _______________ (moderate) corroded engine is a 70 BN cylinder oil.

-- CalSil is a good _______________ (insulate) compared to glass fiber.

-- The rapid _______________ (grow) of piracy attacks in Somali waters is connected to political _______________ (stable) in the area.

-- Is inert gas system _______________ (operation)?

-- The valve to starting air _______________ (distribute) is closed.

-- ______________ (sulphur) acid is formed due to the ________________ (absorb) of the ______________ (condense) present in the combustion process.

-- Insulation is critical for personnel ______________ (protect), _______________ (reduce) of ambient heat or fire _____________ (prevent).

-- _______________ (form) of piston rings and _______________ (align) may cause cylinder oil film deficiencies.

3.  Choose the correct answer.     (15 p.)

-- Whether using a centrifuge or a simple filter, oil cleaning and filtration will be the most effective when the oil is at a:

a. high temperature and a low viscosity

b. low temperature and a high viscosity

c. low temperature and a low viscosity

d. high temperature and a high viscosity

-- Diesel engine lube oil diluted with diesel oil is indicated by:

a. decreased viscosity 		b. decreased pour point 	c. increased flash point

d. increased viscosity

-- Which of the following operations will have a direct impact on the rate of wear in a cylinder liner?

a. temperature of the scavenging air 		b. compression ratio of the piston

c. quality of fuel injected 			d. amount of scavenge air in the cylinder

-- Heavy residual fuel oils are heated prior to centrifuging to:

a. increase specific gravity 			b. separate fuel from lube oil

c. reduce fuel weight 				d. reduce fuel viscosity

-- “Loop”, “uniflow”, “cross flow” are terms used to describe various types of:

a. scavenging      b. turbochargers      c. control air circuits 	   d. supercharging

-- A decrease in the flash point of the diesel engine lube oil indicates that the lube oil is:

a. contaminated with carbon	 		b. diluted with water

c. diluted with fuel oil 		 	d. contaminated with sludge

-- A piston is said to be at top dead centre when it is:

a. closing the fuel ports 		         b. farthest from the cylinder head

c. nearest to the cylinder head		         d. opening the exhaust ports

-- A scored diesel engine cylinder liner will cause:

a. rapid wear of piston rings    b. combustion gases in the cooling water

c. high firing pressure		 d. abnormally high cooling water temperature

-- Which of the listed substances can be satisfactorily removed from diesel oil by centrifuging?

a. gasoline	    b. fuel oil             c. lube oil          d. sludge

-- Combustion knock will most likely occur as a result of using a fuel with:

a. low ignition quality		b. high volatility 	c. low ignition delay

d. a high cetane number

-- A disadvantage of a 2-stroke/cycle diesel engine is:

a. higher working temperatures of the piston and cylinder	b. the use of scavenge air

c. more power strokes per revolution		d. more complicated valve gear

-- A diesel engine which is rated for normal operation at a crankshaft speed of 800 rpm is commonly classed as:

a. constant-speed 	b. slow-speed 	c. high-speed 		d. medium-speed

-- The ignition quality of a diesel oil is indicated by the:

a. octane number 	b. CCAI	c. kinematic viscosity 	d. cetane number

-- Which of the following statements concerning cylinder liner wear is true?

a. Liner wear is normally greatest in the middle of the cylinder

b. Liner wear is distributed equally between the upper and lower portions of the cylinder

c. Excessive but uniform liner wear will not cause wear between piston rings and grooves

d. Excessive liner wear causes wear between piston rings and grooves

-- Burning fuel with a high sulphur content in a diesel engine will:

a. cause clogging of the fuel system

b. increase thermal efficiency

c. increase the ability of the engine to start in cold weather

d. cause corrosion in the cylinder and exhaust systems at low loads

4.  Match the words from the list to their synonyms/definitions below.  (15 p.)

fouled,     downtime,    splinter,    contaminants,   dilute,   slow-steaming,    volatility,     composite,

sluggishness,     fabricate,     perishable,     modifications,    implement,     last,    maladjusted

-- time during which a machine is out of order:

-- wrongly fitted:

-- slow motion, idleness:

-- have a duration:

-- break into small, sharp pieces:

-- made up of several parts or elements:

-- alterations:

-- manufacture:

-- apply:

-- dirty:

-- sensitive:

-- pollutants:

-- the property of changing readily from liquid to vapour/gas:

-- lower fuel consumption during low-load operation:

-- make a liquid thinner by adding water or other solvent:

5.  Choose the correct refrigeration system component. (8 p.)

-- The relief/solenoid/expansion valve provides automatic opening and closing of the liquid lines.

-- The condenser/compressor/receiver is used to liquefy the refrigerant and to subcool it below the saturation temperature by circulating sea water, fresh water or air.

-- The receiver/compressor/evaporator raises the pressure of the vapourised refrigerant causing its temperature to rise so that it becomes higher than that of the refrigerant.

-- The relief/solenoid/expansion valve is controlled by a thermostat to regulate the flow of the refrigerant from the H.P. side of the system to the L.P. side.

-- The evaporator/drier/condenser ensures that the refrigeration system stays dry and clean by removing any moisture and/or contaminants from the refrigerant.

-- The receiver/compressor/evaporator cools down the air in the cargo hold or room by transferring latent heat to the refrigerant.

-- The oily water separator/purifier/oil separator removes oil traces from the refrigerant and ensures that the oil returns to the compressor oil pump.

-- The refrigerant tank/receiver/sump tank serves to collect the cooled refrigerant, and is also used to drain out the refrigerant from the system for maintenance purposes.

6.  Complete the text using the past participles below. (12 p.)

corrected,      increased,     replaced,      placed,      taken,      maintained

gauged,     inspected,     specified,     minimized,     cleaned,     overloaded,

Cylinder liner wear can be ___________, if proper jacket temperature is ____________

The cylinder liner must be ____________ regularly, as it is _______________ in the manual. The liner must be ______________ and ________________ before the gauging. If the temperature of the micrometer exceeds that of the liner, or vice versa, then the readings have to be _______________. The liner wear is measured by a standard template, which consists of strategically positioned holes, in which the micrometer is ______________ and the readings are _______________. The wear rate of the liner will be _________________, if the engine is _________________. Generally, the liner has to be _________________, when the wear is about 0.6-0.8 per cent of the bore diameter.

7.  Complete the sentences with an appropriate preposition. There is one (1) extra preposition. You can choose from the following:  (15 p.)

in,   by,    between,    at,   on,   to,  with,   before,   under,    up

-- Excessive liner wear will cause increased blow-__________________.

-- Treat oil spill _______________ dispersants.

-- Even engines operating _______________ low sulphur fuels may be prone _______________ scuffing damage.

-- The spillage has been stopped and cleaned ______________.

-- When the engine runs unevenly and will not pick ____________ rpm, the fuel filter may be blocked.

-- Stand _______________ oil clearance team and report.

-- Most types of cylinder liner wear take place _______________ the piston rings and the liner.

-- There is leak _______________ manifold connection.

-- HFO has to be heated prior ______________ centrifuging.

-- One of the main purposes of a ship's refrigeration system is to keep the cargo _______________ good condition.

-- Permanent insulation is cut and fitted _______________ site.

-- In compliance _______________ Tier II NOx regulations and Energy Efficiency Design Index (EEDI) guidelines, engine cylinders must operate _______________ increased pressures and reduced operating temperatures. These regulations have led to an increase _______________ cold corrosion.

GOOD LUCK!!!